ASIAN SCHOOL OF MANAGEMENT AND TECHNOLOGY

[Affiliated to Tribhuvan University]

ASSIGNMENT: KIT LAB REPORT OF MICROPROCESSOR

Submitted By                                                                                   Submitted To:

Name: Manalika Shrestha                                                            Er. Manoj Giri

Roll No: 13                                                                                       ____________________

Program: B.Sc. CSIT

Submission Date:    /    /2080

Registration No:

Kathmandu, Nepal

Table of contents                                                    Page

| 1. WAP to add 56h and 37h and store the sum in memory address 8050h. | 3 |
| --- | --- |
| 2. WAP to add two number stored in 8050h and store sum in 8052h. | 4 |
| 3. WAP to subtract 37h from 52h and store the difference in memory address 8050. | 5 |
| 4. WAP to find the difference of number stored in location 9050 and 9051 and stored the difference in 9052. | 6 |
| 5. WAP to find greater number between two numbers stored in location 8050 and 8051 and stored the bigger number in memory address 9050. | 7 |
| 6. WAP a program to find a given number is positive or negative and if negative stored in location 8051 and positive in 8050. | 8 |
| 7. WAP to find a given number is even or odd and if even store in location 9050h and if odd store in 9051h. | 10 |
| 8. WAP to add two 16 bits numbers. | 12 |
| 9. WAP to add two numbers stored in location 8000 and 8001 and store the sum in 8003 and carry in 8002. | 13 |
| 10. WAP to multiply two number stored in location 8002 and 8003 and stored the product in 8084. | 14 |
| 11. WAP to find quotient and remainder and store quotient at location 8050 and remainder at location 8051. | 15 |
| 12. Find compliment of a number store in the location 9051and store its compliment in location 9052. | 17 |
| 13. Find compliment 2’s of a number stored in location 8050 and store its compliment at location. | 18 |
| 14. Find the greatest number out of 10 number stored from location 9010 and number stored the greatest number at location 9060. | 19 |
| 15. Find the greatest number out of 10 number stored from location 9010 and store the smallest at location 9060. | 21 |
| 16. WAP to copy 10 data from memory location 8050 to new memory location 9050. | 23 |
| 17. WAP to generate 10 even numbers. | 25 |
| 18. WAP to generate 10 odd number. | 26 |
| 19.WAP to count even number or odd number out of 10 number and store even number in location 8050H and odd number in location 8060H. | 27 |
| 20.WAP to sort given 10 numbers from memory location 8200H in ascending order. | 29 |

1.Write a program to add 56H and 37H and store the sum in memory address 8050H.

MVI A, 56H

ADI 37H

STA 8050H

HLT

OUTPUT:

2.Write a program to add two number store in 8050H and 8051H and store the sum in 8052H.

LDA 8050H

MOV H, A

LDA 8051H

ADD H

MOV L, A

MVI A, 00

MOV H, A

SHLD 8052H

HLT

OUTPUT:

3.Write a program to subtract 37H from 52H and store the difference in memory address 8050.

MVI A, 52H

SUI 37H

STA 8050

HLT

OUTPUT:

4.Write a program to find the difference of number stored in location 9050 and 9051 and stored the difference in 9052.

LDA 9050

M0V B, A

LDA 9051

SUB B

STA 9052

HLT

OUTPUT:

5.Write a program to find greater number between two number stored in location 8050H and 8051H and stored the bigger number in memory address 9050H.

LDA 8050H

MOV B, A

LDA 8051

CMP B

JNC GREATER

MOV A, B

GREATER: STA 9050

HLD

OUTPUT:

6. Write a program to find a given number is positive or negative and if positive store in location 8050 if negative store in 8051.

LDA 9050H

MOV B, A

RAL

JC NEGATIVE

MVI A,00H

RAR

MOV A, B

STA 8050H

NEGATIVE: MOV A, B

STA 8051H

HLT

OUTPUT:

7.Write a program to find a given number is even or odd and if even stored in location 9050H and if odd stored in 9051H.

LDA 8050H

ANI 01H

JZ EVEN

MVI A, 01H

STA 9050H

EVEN: MVI A, 00H

STA 9050H

HLT

OUTPUT:

8.Write a program to add two 16 bits numbers stored in location 9050,9051 and 9052,9053 and then stored the sum in 9054 and 9055.

LHLD 9050

XCHG

LHLD 9052

DAD D

MVI C,00H

JNC LOOP

INR C

LOOP: SHLD 9054

MOV A, C

STA 9056

HLT

OUTPUT:

9.Write a program to add two number stored in location 8000 and 8001 store the sum of addition in 8003 and carry in 8002.

MVI C,00

LDA 8000H

MOV B, A

LDA 8001H

ADD B

STA 8003H

INR C

MOV A, C

STA 8002H

HLT

OUTPUT:

10. WAP to multiply two number stored in location 8002 and 8003 and stored the product in 8084.

LDA 8000

MOV D, A

LXI H, 8003

MOV C, M

BACK: DCR C

JZ SKIP

ADD D

JMP BACK

SKIP: STA 8004

HLT

OUTPUT:

11.WAP to find quotient and remainder and store quotient at location 8050 and remainder at location 8051.

LXI H,8000

MOV B, M

MVI C,00

INX H

MOV A, M

NEXT: CMP B

JC LOOP

SUB B

INR C

JMP NEXT

LOOP: STA 8081

MOV A, C

STA 8050

HLT

OUTPUT:

12. Find compliment of a number store in the location 9051and store its compliment in location 9052.

LDA 9051

CMA

STA 9052

HLT

OUTPUT:

13. Find compliment 2’s of a number stored in location 8050 and store its compliment at location.

LDA 8050

CMA

INR A

STA 8051

HLT

OUTPUT:

14. Find the greatest number out of 10 number stored from location 9010 and number stored the greatest number at location 9060.

LXI H,9010

MOV C, M

INX H

DCR C

MOV A, M

Skip: INX H

CMP M

JNC LOOP

MOV A, M

LOOP:  DCR C

JNC Skip

STA 9060

HLT

OUTPUT:

15.Find the smallest number out of 10 number stored from location 9010 and store the smallest at location 9060.

LXI H, 9010

MOV C, M

INX H

MOV B, M

DCR C

LOOP: INX H

MOV A, M

CMP B

JNC Skip

MOV B, A

Skip: DCR C

JNZ LOOP

LXI H,9060H

MOV M, B

HLT

OUTPUT:

16. WAP to copy 10 data from memory location 8050 to new memory location 9050.

LXI H,8050H

LXI D,9050H

MVI B,10H

LOOP:MOV A, M

STAX D

INX H

INX D

DCR B

JNZ LOOP

HLT

OUTPUT:

17. WAP to generate 10 even numbers.

MVI A,00H

MVI B,02H

MVI C, 0AH

LXI H,9200H

LOOP: ADDB

MOV M, A

INX H

DCR C

JNZ LOOP

HLT

OUTPUT:

18. WAP to generate 10 odd number.

MVI A,01H

MVI B,02H

MVI C,0AH

LXI H, 9050H

LOOP: ADD B

MOV M, A

INX H

DCR C

HLT

OUTPUT:

19.WAP to count even number or odd number out of 10 number and store even number in location 8050H and odd number in location 8060H.

LXI H,8000H

MOV C, M

MVI B,00H

MVI D,00H

LOOP2: INX H

MOV A, M

RAR

JC LOOP1

INR B

JMP LOOP3

LOOP1: INR D

LOOP3: DCR C

JNZ LOOP2

MOV A, B

STA 8050h

MOV A, D

STA 8060h

HLT

OUTPUT:

20.WAP to sort given 10 numbers from memory location 8200H in ascending order.

MVI B, 09

START: LXI H, 8200H

MVI C,0AH

BACK: MOV A, M

INX H

CMP M

JNC SKIP

MOV D, M

MOV M, A

DCX H

MOV M, D

INX H

SKIP: DCR C

JNZ BACK

DCR B

JNZ START

HLT

OUTPUT: